室温计

武伯熹 3120104523

一、实验目的

学习uC/OS II的应用程序编写；

理解如何直接操纵GPIO，体会与Linux的不同；

学习单总线设备的访问方式；

学习7段数码管的时分复用驱动方式。

二、实验器材

硬件

- pcDuino v2板一块；

- 5V/1A电源一个；

- microUSB线一根；

- 面包板一块；

- 两位7段数码管（共阳）一颗；

- 360Ω 1/8W电阻2颗；

- DHT-11 温湿度传感器1个；

- 面包线若干。

以下为自备（可选）器材：

- PC（Windows/Mac OS/Linux）一台；

- USB-TTL串口线一根（FT232RL芯片或PL2303芯片）；

- 以太网线一根（可能还需要路由器等）；

- 1602 LCD（带配套的5k微调电阻）。

软件

- 编译软件；

- Fritzing

三、实验步骤

设计输出方案，画连线示意图；

在面包板上连线，完成外部电路；

编写C/C++程序，测试程序和电路；

测试、实现uC/OS II对GPIO的访问；

实现DHT-11数据的读；

实现以时分复用方式在四位7段数码管上依次显示0000-9999的数字；

用两个uc/OS II任务，一个定时读DHT-11数据，一个轮流驱动数码管，一秒一次显示当 前温度和湿度。注意处理 好两个任务之间的数据共享。

四、实验结果

1.画出你所实际实施的连接示意图；

2. 用Fritzing画出外部设备的连线图，附实物照片；

3.描述所做的实验步骤，给出各步操作的命令和结果；

获取wiringPi

build

check

4.给出代码并解释；

#include <wiringPi.h> #include <stdio.h> #include <stdlib.h> typedef unsigned char uint8; typedef unsigned int uint16; typedef unsigned long uint32; #define HIGH_TIME 32 int pinNumber =1; //use gpio1 to read data uint32 databuf; uint8 readSensorData(void) { uint8 crc; uint8 i; pinMode(pinNumber,OUTPUT); // set mode to output digitalWrite(pinNumber, 0); // output a high level delay(25); digitalWrite(pinNumber, 1); // output a low level pinMode(pinNumber, INPUT); // set mode to input pullUpDnControl(pinNumber,PUD_UP); delayMicroseconds(27); if(digitalRead(pinNumber)==0) //SENSOR ANS { while(!digitalRead(pinNumber)); //wait to high for(i=0;i<32;i++) { while(digitalRead(pinNumber)); //data clock start while(!digitalRead(pinNumber)); //data start delayMicroseconds(HIGH_TIME); databuf*=2; if(digitalRead(pinNumber)==1) //1 { databuf++; } } for(i=0;i<8;i++) { while(digitalRead(pinNumber)); //data clock start while(!digitalRead(pinNumber)); //data start delayMicroseconds(HIGH_TIME); crc*=2; if(digitalRead(pinNumber)==1) //1 { crc++; } } return 1; } else { return 0; } } int main (void) { printf("Use GPIO1 to read data!\n"); if (-1 == wiringPiSetup()) { printf("Setup wiringPi failed!"); return 1; } pinMode(pinNumber, OUTPUT); // set mode to output digitalWrite(pinNumber, 1); // output a high level printf("Enter OS-------\n"); while(1) { pinMode(pinNumber,OUTPUT); // set mode to output digitalWrite(pinNumber, 1); // output a high level delay(3000); if(readSensorData()) { printf("Congratulations ! Sensor data read ok!\n"); printf("RH:%d.%d\n",(databuf>>24)&0xff,(databuf>>16)&0xff); printf("TMP:%d.%d\n",(databuf>>8)&0xff,databuf&0xff); databuf=0; } else { printf("Sorry! Sensor dosent ans!\n"); databuf=0; } } return 0; }

5.将所做作品拍摄视频上传到优酷，给出优酷的视频网址；

http://v.youku.com/v_show/id_XMTI2NjY2MTY3Ng==.html?from=y1.7-1.2